# CHAPTER 1

INTRODUCTION

  - Project Background

### The Government of Karnataka, Public works, ports and inland water transport department, Bangalore has contemplated for "The Construction of 4 lane and 2 lane road in PWD, Bangalore Circle, Bangalore" in Bangalore, Bangalore Rural, Kolar, Chikkaballapura, Tumkur and Ramanagara Districts, in the state of Karnataka.

  - The Project Highway

The Government of Karnataka, Public works, ports and inland water transport department, Bangalore has contemplated for Widening to 4 Lane (Road connecting District to District) of

Vijayapura Nandi road via Devanahalli Kempapura (SH-96),Chikkaballapura, Bangalore Rural and  Kolar District. The total Length of the Project Road is 54.0 Km. A location map of the project road is shown in the figure 1.1. The Alignment of the project road starts at NH-7 near Doddamarali village and runs through nearby villages namely Vijayapura, Jangamakote, H-Cross, Vemgal and ends at Kolar @ CH 53+500 Km of SH-5 (Overlap of SH-96).

  - Objective of the Consultancy Services

The objective of this consultancy is to undertake feasibility studies and prepare a Detailed Project Report of the Project Highway for the purpose of firming up the Authority’s requirements in respect of development and construction of the Project Highway and Project Facilities and enabling the prospective bidders to assess the Authority’s requirements in a clear and predictable manner with a view to ensuring:

- Enhanced safety and level of service for the road users

Figure 1- 1

: Location map of the project Road

- Superior operation and maintenance enabling enhanced operational efficiency of the Project Highway

- Minimal adverse impact on the local population and road users due to road construction

- Minimal adverse impact on environment

- Minimal additional acquisition of land

- Phased development of the Project Highway for improving its financial viability consistent with the need to minimise frequent inconvenience to traffic that may be caused if additional works are undertaken within a period of seven years from the commencement of construction of the Project Highway.

  - Scope of Consultancy Services

The scope of service shall comprise:

- Traffic surveys and demand assessment

- Engineering surveys and investigations

- Location and layout of truck lay byes

- Location and layout of bus bays and bus shelters

- Preparation of designs for road, (Pavement/Geometric Design)

- Preparation of designs for bridges for (Old and New, ‘x’ number of Major/Minor bridges) and bridge components.

- Preparation of Land Plan Schedules and Utility Relocation Plans

- Preparation of BOQ and Cost Estimates

- Final Detailed Project Report and final drawings.

  - Progress Achieved

The Project Consultants Team members have visited the Projects road, carried out extensive inventory and visual condition survey of existing road alignment, road pavement, shoulder, major junctions, minor junctions, accident prone locations and collected other existing features and other relevant secondary data. The team members were also visited the Office of the Superintending engineer, Office of the Executive Engineer, Bangalore and interacted with concerned Section Officers and Assistant Executive Engineers. The Team Leader has also visited the project site and discussed the salient features of project road with the concerned Executive Engineer, Public works, ports and inland water transport department, Kolar. The team has made site visits along the project corridor to gather primary information on land use, traffic characteristics, soil characteristics, pavement as well as structure conditions, intersections and status of land availability along the same. We believe that, this knowledge regarding project site will definitely help us in providing sufficient inputs for the formulations of an engineering and sequential approach and methodology for the proposed study.  This will further enable us to address the problem in a rational and logical manner. The following progress has been achieved so far.

i)    Road, Pavement and Shoulder Inventories and visual conditions survey - completed

ii)   Major Junctions, Minor Junctions and other intersections survey - completed

iii)  Identification of Traffic Survey locations - Completed

iv)  CD Structures inventories and visual condition survey - Completed

v)   Topo Survey and Level Survey - Completed

vi)  Traffic Volume Count and TMC Survey - Completed

vii) Axle Load Survey - Completed

viii) Pavement Investigations - Trial Pits, recording of existing pavement layer crest

thicknesses, field Density, Field Moisture content, Soil Sampling - Completed

ix) Program of Inventory, Visual Condition Survey, Investigations and Reports preparation /

Submission - Completed

x)  Inception Report - Submitted

The Present Submission is the Report on Alignment and First Traffic survey

# CHAPTER 2

DESIGN STANDARDS AND ALIGNMENT PROPOSALS

2.1 General

A highway system development involves huge irreversible investments, and requires rigorous modelling and analysis before the implementation decision is made. Objectives of setting design standards is to confirm the proposed road with the codes of practice, to give a clear picture of how the proposed facility would look like once implemented and also to maintain uniformity throughout the corridor to achieve the desired purpose of the facility and to ensure safety and comfort for the road users. Non-uniform or inadequate design standards lead to confusion for the drivers to adapt to the changing road characteristics and cause accidents. For the present Project it is proposed to follow design standards given in Indian Roads Congress (IRC) codes of practice, guidelines and special publications, as well as applicable MoRT&H circulars and reports. Where the said standards are silent for any particular element of design, other standards, like, American Association of State Highway and Transport Officials (AASHTO) Standards and Any other National or International Standard as considered suitable shall be referred to and the one considered the best and relevant adopted.

## 2.2 Geometric Design

## 2.2.1 Base Plan

Base plan of the existing project road showing all natural and manmade features has been prepared using the topographical survey data collected through Total Station Survey. All the features within a band width of 60m have been captured with a unique “description code” during the topographical survey along with the details of existing carriageway centreline, edge of pavement, edge of shoulder, toe line of the embankment etc. This data has been downloaded into the software environment to prepare the base plan. The following activities elucidate the preparation of base plans in more details:

- Download the data into software

- Define main corridor features by joining the points of centreline, edge of pavement, embankment toe line

- Join the points with same description codes for all physical features like rivers, buildings, religious structures, shops, telephone poles, electric poles, cross roads etc within the above specified limits

- Establish break lines for features such as edge of the road, shoulder, nalas, top and bottom of ditches, etc;

- Insert the details of existing cross drainage structures such as bridge number, span arrangement etc.

- Insert details of underground utility services collected from secondary sources.

- Cross check the so prepared base plans by “walkover” surveys

- Update and finalize the base plans with additional survey data if necessary.

References

IRC: 38-1988

IRC: 86-1983

AASHTO-Green Book

IRC: SP-73-2007

2.2.2 Geometric Design Standards

## 2.2.2.1 General

Planning and design of roads & CD structures, pedestrian facilities and street furniture are essentially based on IRC standards and MoRT&H specifications. Whenever, the codes/standards are silent on some of the aspects, the same will be planned/designed based on sound engineering practices and consultant’s experience in similar projects. Design standards relevant to the project road along with the broad list of design parameters and the relevant IRC codes/specifications have been detailed in Table 2.1

The design standards corresponding to 80 - 100 kmph have been adopted in general for the project road to ensure uniform and consistent design standards for safety reasons. Locations of horizontal curves have been checked for the adequacy of radius for the set design speed. Most of the  curves satisfy  the requirements  of  design  standards  for a  design  speed  of

80 - 100 kmph.

Table 2- 1:   Relevant IRC Codes / Specifications

| Sl. No. | Design Parameters | Reference Code |
| --- | --- | --- |
| 1 | Geometric Design Standards | As per IRC: 38-1988, IRC: 73-1980, IRC: 86-1983 & IRC:SP: 23-1983 |
|  |  Carriageway width |  |
|  |  Shoulder |  |
|  |  Footpath |  |
|  |  Median |  |
|  |  Camber |  |
|  |  Super elevation |  |
|  |  Horizontal Alignment |  |
|  |  Vertical Alignment |  |
| 2 | Road Furniture |  |
|  |  Road Markings | As per IRC:79-1981 |
|  |  Road Signs | As per IRC:35-1997 |
|  |  Road Delineators | As per IRC:67-2001 |
| 3 | Pedestrian Facilities | As per IRC:103-1988 |
| 4 | Road Capacity | As per IRC:64-1990 |
| 5 | Traffic Forecast | As per IRC:108-2015 |
| 6 | Pavement Design | As per IRC:37-2012 |
| 7 | Overlay Design | As per IRC:81-1997 |
| 8 | Road Drainage | As per IRC:SP:42-1994 & IRC:SP:50-1999 |
| 9 | Culverts & Bridges | As per relevant IRC codes and MORT&H |

Table 2- 2: Design Standards

| Sl. No. | Description | Standards |
| --- | --- | --- |
| 1 | ROAD LAND WIDTH |  |
|  | a. Right of Way | 30 - 60m |
|  | b. Set Back Between Building Line and Road Boundary | 3m to 6m |
| 2 | Design Speed | Desirable 100 kmph |
|  |  | Minimum 80 kmph |
| 3 | Main carriageway |  |
|  | a. Traffic lane width | 3.5m |
|  | b. Paved Shoulder width |  |
|  | In rural areas | - |
|  | In urban/semi urban areas | - |
|  | c. Earthen Shoulder Width |  |
|  | In rural areas | 1.0m |
|  | In urban/semi urban areas | - |
|  | d. Cross Slope |  |
|  | In Bituminous surface | 2.50% |
|  | In Hard shoulder | 3.00% |
|  | e. Super elevation | Maximum 7.0% |
|  | f. Rate of Change of Super elevation | 1 in 150 |
| 4 | Horizontal Curves |  |
|  | a. Requiring no super elevation (for 100kmph) | 2000m |
|  | b. Requiring no super elevation (for 80kmph) | 1200m |
|  | Super Elevation |  |
|  | a. Absolute minimum requiring 7% super elevation (for 80kmph) | 360 m |
|  | b. Absolute minimum requiring 7% super elevation (for 65kmph) | 240 m |
| 5 | Vertical curves |  |
|  | a. Minimum length of Vertical Curve | 50m |
|  | b. Maximum Grade Change not Requiring Vertical Curve | 0.60% |
|  | c. Minimum ‘k’ value for summit curves | 73.6 |
|  | d. Minimum ‘k’ value for valley curves | 41.5 |
| 6 | Gradient |  |
|  | a. Maximum | 3.30% |
|  | b. Minimum | 0.30% |
|  | In cut and kerbed sections | 0.50% |
|  | On un kerbed sections on embankment | 0.30% |
| 7 | Sight Distance |  |
|  | (Sight distance shall be in accordance with the intermediate sight criterion preferably but in no case shall be less than the stopping sight distance) |  |
|  | a. Desirable (intermediate) | 360m |
|  | b. Minimum (stopping) | 180m |

## 2.3 Capacity Standards

References

IRC: 64-1990

IRC: 106- 1990

IRC: 64-1990 is the main reference for determining capacity standards for roads in India. The following Table summarises the capacity standards and design service volumes for various categories of roads in plain areas for the peak hour traffic of 6-8% and design service level corresponding to level of service-B (LoS B).

Table 2- 3:   Capacity Standards

| Type | Design Service Volume PCUs/day |
| --- | --- |
| 2- lane carriageway (Plain Terrain) |  |
| Without Paved Shoulders | 15,000 |
| With minimum 1.5m Paved Shoulders | 18,000 |
| 4 Lane carriageway (Plain Terrain) | 40,000 |
| 4 Lane Carriageway (Rolling terrain) | 20,000 |

## 2.4   Transition Curves

As per IRC: 73-1980, Geometric Design Standards for Rural Highways, suggests that the length of the transition curve should be the larger of the two values arrived at on the basis of the following criteria:

- Rate of change of centrifugal acceleration; and

- Rate of change of super elevation (not steeper than 1 in 150)

The values given in the IRC Geometric Design Standards for Rural Highways are obtained from the criterion of rate of change of centrifugal acceleration, with a pavement width of 7.00m. The transition lengths so obtained for various radii are presented in table below.

Table 2- 4: Transition Lengths for different Design Speeds

| Radius (m) | Transition Length (m) 100 kmph | Transition Length (m) 80 kmph |
| --- | --- | --- |
| 45 | - | - |
| 60 | - | - |
| 90 | - | - |
| 100 | - | - |
| 150 | - | - |
| 170 | - | - |
| 200 | - | NA |
| 240 | - | 90 |
| 300 | NA | 75 |
| 360 | 130 | 60 |
| 600 | 80 | 35 |
| 700 | 70 | 35 |

| Radius (m) | Transition Length (m) 100 kmph | Transition Length (m) 80 kmph |
| --- | --- | --- |
| 800 | 60 | 30 |
| 900 | 55 | 30 |
| 1000 | 50 | 30 |
| 1200 | 40 | NR |
| 1500 | 35 |  |
| 1800 | 30 |  |
| 2000 | NR |  |

2.5   Design Standard of Bridges, Culverts & Structures

The Design Standards and the loading to be considered are generally based on the requirements laid down in the latest editions of IRC/ IS codes of practices & standard specifications, and guidelines of Ministry of Surface Transport. Additional technical references would be used wherever the provisions of IRC/IS codes are found inadequate.

Following IRC/IS codes are proposed to be used in the design

For items not covered in the above specifications, provisions of following standards will be followed in the given order of priority:

- Provisions of IS codes of practices; and

- Relevant provisions of BS codes of practices

Sound Engineering Practice, Technical Literatures/ Papers & provision of relevant codes of other nations.

Table 2- 5: Design Standards

| Sl. no | CD STRUCTURES (Bridges & Culverts) | Standards |
| --- | --- | --- |
| 1 | Culverts, Clear width for new 2-lane Without service road | 15 m |
| 2 | Bridges (without footpath) outer to outer parapets For 2-lane bridge | 13 m |
| 3 | Width of Safety Kerb | 750 mm |
| 4 | Width of Crash barrier | 450mm |
| 5 | Width of Footpath and Railing | 2225 mm |

# CHAPTER 3

TRAFFIC SURVEY AND ANALYSIS

## 3.1   General

The Alignment of the project road starts at NH-7 near Doddamarali village and runs through nearby villages namely Vijayapura, Jangamakote, H-Cross, Vemgal and ends at Kolar @ CH 53+500 Km of SH-5 (Overlap of SH-96).

The salient features of the Project Road are detailed below.

Table 3- 1: Salient Features of the Project Road

| Sl. No | Description | Details |
| --- | --- | --- |
| 1 | Name of the Road | Vijayapura Nandi road via Devanahalli Kempapura (SH-96) |
| 2 | Type of the Road | MDR, SH- 96 |
| 3 | Start of the Project (NH- 7) | Ch: 0+000 Km |
| 4 | End of the Project (Kolar) | Ch: 53+500 Km |
| 5 | Length of the Project Road | 54.0 00 km |
| 6 | Proposed Length of the Project Road after Re-alignment and Curve improvements | 53.500 km |
| 7 | Right Of Way (ROW) | Varies From 8 m to 30 m |
| 8 | Proposed Lane Configuration | 4 - Lane |
| 9 | Proposed Design Speed | 80-100 Kmph |
| 10 | CD Structures |  |
|  | Existing Structues | Two Lane |
|  | Pipe Culvert | 5 |
|  | Stone Culvert | 13 |
|  | Major Bridge | 10 |
|  | Minor Bridge | 9 |
|  | RCC Slab | 28 |
|  | Total | 65 |
| 11 | Junctions |  |
|  | Major Junctions | 4 |
|  | Minor Junctions | 26 |
|  | Total | 30 Nos. |
| 12 | City/ Villages along the project stretch | Vijayapura, Venkatapura, Jangamakote, H-cross, Vemgal |
| 13 | Major Connectivity | It connects the NH 7, SH 5, SH 35, SH 82 and SH 99. |
| 14 | Project Safety Features - Proposed | Road Marking, Sign Boards, Road Side Safety Barriers and Safety During Construction |
| 15 | Proposed Project Facilities | Pedestrian Footpath, Street Lighting, Truck Lay Byes, Bus Shelters, Junction improvements etc |

## 3.2   Traffic Surveys

To assess the present day traffic characteristics, primary surveys are intended to understand the existing traffic scenario on the project road. The following surveys were carried out:

- Classified Volume counts

- Turning Movement Volume Count (TMC) at Major junctions

- Axle Load Surveys

The above all traffic surveys has been carried out according to the ToR mandate and analysed as per IRC codes.

Table 3- 2: Details of Traffic Survey and Locations

| Type of survey | Chainage | Location | Survey dates |  | Duration |
| --- | --- | --- | --- | --- | --- |
|  |  |  | From | To |  |
| Classified Volume Counts | 9+500 | Chikkanahalli | 16/08/2015 | 22/08/2015 | 7 days X 24 Hours |
|  | 27+000 | Near H-Cross | 10/08/2015 | 16/08/2015 |  |
|  | 40+000 | Vemgal | 10/08/2015 | 16/08/2015 |  |
| Turning Volume Counts | 12+000 | Shivganesh circle | 19/08/2015 |  | 8:00 AM to 12:00 PM and 04:00 PM to 08:00 Pm (8 Hours) |
|  | 18+000 | Jangamakote cross | 20/08/2015 |  |  |
|  | 27+500 | H-Cross | 21/08/2015 |  |  |
| Axle Load Survey | 9+500 | Chikkanahalli | 22/08/2015 |  | 1 Day (24 Hours) |
|  | 27+000 | Near H-Cross | 21/08/2015 |  |  |

## 3.3   Methodology for Traffic Surveys

## 3.3.1   Classified Traffic Volume Counts

Hourly Traffic Counts Were Conducted For Seven Day (24 Hours) simultaneously in both directions at each survey Location to obtain data on The Magnitude of Traffic flow, Hourly Variation and the Traffic Composition. The survey was carried out by manual vehicle counting and classifying the vehicles passing the survey station in both directions, in 15 minutes intervals. The counts were made separately for motorized and non-motorized traffic as per the vehicle classification system.

The volume count data was processed and compiled using Excel Spread Sheet software packages to get hourly and direction-wise classified traffic data.

## 3.3.2     Intersection Turning Movement Count Survey (TMC)

The surveys were conducted for one day (8 hours) to obtain classified turning movement at the identified junctions. The data was collected by deploying enumerators to record data for each hour within fifteen minutes intervals. All the surveys were conducted through trained enumerators and were supervised by traffic engineers.

## 3.4      PCU factors Adopted For the Study as Per IRC: 64 – 1990

Since most of the length of project corridor falls in the rural are, the PCU’s applicable for rural section has been adopted. PCU values adopted for the present study are as per IRC: 64 – 1990 “Guidelines for the Capacity of Rural Roads” recommended for rural roads.

Table 3- 3: PCU factors adopted for the study as per IRC: 64 - 1990

| Sl. No. | Vehicle Type | PCU |
| --- | --- | --- |
| 1 | Two wheeler | 0.5 |
| 2 | Car | 1 |
| 3 | Auto | 1 |
| 4 | Bus | 3 |
| 5 | LCV | 1.5 |
| 6 | 2 Axle Truck | 3 |
| 7 | 3 Axle Truck | 3 |
| 8 | MAV | 4.5 |
| 9 | Cycle | 0.5 |
| 10 | Tractor | 4.5 |

## 3.5     Data Analysis

## 3.5.1    Classified Volume Count Characteristics

Classified traffic volume count data has been analysed with respect to traffic intensity, hourly variation of traffic and composition.

## 3.5.1.1     Average Daily Traffic

Classified traffic volume counts data has been analyzed with respect to traffic intensity, hourly variation of traffic and composition. The details are shown in Table 3.4.

- The total volume of vehicular traffic is observed near Chikkanahalli (CH: 9+500 Km) Direction: Chikkaballapura to Kolar is 3,274 (3,330 PCU’s) and Kolar to Chikkaballapura is 3,260 (3,345 PCU’s) and total vehicles observed were 6,533 (6,676 PCU’s).

- The total volume of vehicular traffic is observed near H-cross (CH: 27+000 Km) Direction: Chikkaballapura to Kolar is 5,450 (6,077 PCU’s) and Kolar to Chikkaballapura is 6,071 (6,855 PCU’s) and total vehicles observed were 11,521 (12,931 PCU’s).

- The total volume of vehicular traffic is observed near Vemgal (CH: 40+000 Km) Direction: Chikkaballapura to Kolar is 3,645 (3,586 PCU’s) and Kolar to Chikkaballapura is 3,656 (3,586 PCU’s) and total vehicles observed were 7,301 (7,172 PCU’s).

Table 3- 4:  Average Traffic Volume count

a) Near Chikkanahalli (CH: 9+500 Km)

| Direction | Truck |  |  | LCV | Minibus / Van / Tempo | Buses |  | Cars / Jeeps | Motor Cycle | Auto | Tractors | Cycles | Carts | Volume | PCU's |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2-Axle | 3-Axle | Multi-Axle |  |  | Govt Buses | Other Buses |  |  |  |  |  |  |  |  |
| Chikkaballapura to Kolar | 57 | 56 | 46 | 340 | 56 | 60 | 26 | 405 | 1954 | 118 | 86 | 69 | 1 | 3,274 | 3,330 |
| Kolar to Chikkaballapura | 55 | 59 | 53 | 349 | 60 | 60 | 28 | 391 | 1932 | 117 | 81 | 74 | 2 | 3,260 | 3,345 |
| Average Daily Traffic (ADT) | 112 | 115 | 99 | 689 | 116 | 120 | 55 | 796 | 3886 | 235 | 167 | 142 | 3 | 6,533 | 6,676 |

b) Near H-cross (CH: 27+000 Km)

| Direction | Truck |  |  | LCV | Minibus / Van / Tempo | Buses |  | Cars / Jeeps | Motor Cycle | Auto | Tractors | Cycles | Carts | Volume | PCU's |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2-Axle | 3-Axle | Multi-Axle |  |  | Govt Buses | Other Buses |  |  |  |  |  |  |  |  |
| Chikkaballapura to Kolar | 302 | 154 | 102 | 331 | 130 | 97 | 55 | 914 | 3008 | 214 | 85 | 50 | 8 | 5,450 | 6,077 |
| Kolar to Chikkaballapura | 364 | 165 | 103 | 427 | 133 | 102 | 60 | 1141 | 3152 | 274 | 93 | 52 | 5 | 6,071 | 6,855 |
| Average Daily Traffic (ADT) | 665 | 319 | 205 | 758 | 263 | 199 | 115 | 2054 | 6160 | 488 | 179 | 102 | 13 | 11,521 | 12,931 |

c) Near Vemgal (CH: 40+000 Km)

| Direction | Truck |  |  | LCV | Minibus / Van / Tempo | Buses |  | Cars / Jeeps | Motor Cycle | Auto | Tractors | Cycles | Carts | Volume | PCU's |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2-Axle | 3-Axle | Multi-Axle |  |  | Govt Buses | Other Buses |  |  |  |  |  |  |  |  |
| Chikkaballapura to Kolar | 110 | 67 | 30 | 196 | 28 | 114 | 39 | 560 | 2225 | 220 | 32 | 14 | 10 | 3,645 | 3,586 |
| Kolar to Chikkaballapura | 99 | 62 | 23 | 212 | 30 | 124 | 36 | 610 | 2170 | 234 | 36 | 12 | 8 | 3,656 | 3,586 |
| Average Daily Traffic (ADT) | 210 | 128 | 53 | 408 | 58 | 238 | 75 | 1170 | 4394 | 454 | 68 | 27 | 18 | 7,301 | 7,172 |

## 3.6 Traffic Forecast

Forecasting the traffic is an important element for planning and design of infrastructure facility, especially when it is proposed to be taken up on a commercial format. The present traffic demand will increase primarily due to the growth in activities earmarked for industrial development besides socio – economic characteristics of the traffic in influencing regions. Estimation of traffic growth rates is further needed for deciding suitable development strategy and assessing the financial viability of the project.

## 3.6.1   Growth of Vehicle Population

The vehicle population in the state is showing an increasing trend. The average number of vehicles registered in the state has been increasing every year. The future growth pattern is likely to be affected by the following factors:

- Recent economic changes taking place in the project influence region

- Future modal shift among the vehicle types, in road traffic as well as between various

modes.

- Development of vehicle technology

- Development of Industries and production of Agricultural products

References

IRC: 37-2012

3.6.2 Suggested Traffic Growth rates for the project road

As per the IRC guidelines, Five percent growth is to be considered for the

Determination of lane requirements/ project preparation and for financial

viability of the project.

Applying the traffic growth rates of 5 % to AADT, traffic projections on the basis of average traffic in two locations for a period of 20 years are presented

Table 3- 5: Traffic Projections of location

a) Near Chikkanahalli (CH: 9+500 Km)

| Year | Truck |  |  | LCV | Minibus / Van / Tempo | Buses |  | Cars / Jeeps | Motor Cycle | Auto | Tractors | Cycles | Carts | Volume | PCU's |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2-Axle | 3-Axle | Multi-Axle |  |  | Govt Buses | Other Buses |  |  |  |  |  |  |  |  |
| 2015 | 112 | 115 | 99 | 689 | 116 | 120 | 55 | 796 | 3886 | 235 | 167 | 142 | 3 | 6533 | 6676 |
| 2016 | 118 | 121 | 104 | 723 | 121 | 126 | 57 | 835 | 4080 | 247 | 175 | 149 | 3 | 6860 | 7009 |
| 2017 | 124 | 127 | 109 | 759 | 127 | 132 | 60 | 877 | 4284 | 259 | 184 | 157 | 3 | 7203 | 7360 |
| 2018 | 130 | 133 | 114 | 797 | 134 | 139 | 63 | 921 | 4498 | 272 | 193 | 165 | 3 | 7563 | 7728 |
| 2019 | 136 | 140 | 120 | 837 | 140 | 146 | 67 | 967 | 4723 | 286 | 203 | 173 | 4 | 7941 | 8114 |
| 2020 | 143 | 147 | 126 | 879 | 148 | 153 | 70 | 1015 | 4959 | 300 | 213 | 182 | 4 | 8338 | 8520 |
| 2021 | 150 | 154 | 132 | 923 | 155 | 161 | 73 | 1066 | 5207 | 315 | 223 | 191 | 4 | 8755 | 8946 |
| 2022 | 158 | 162 | 139 | 969 | 163 | 169 | 77 | 1119 | 5468 | 331 | 235 | 200 | 4 | 9193 | 9393 |
| 2023 | 166 | 170 | 146 | 1017 | 171 | 177 | 81 | 1175 | 5741 | 347 | 246 | 210 | 4 | 9653 | 9863 |
| 2024 | 174 | 179 | 153 | 1068 | 179 | 186 | 85 | 1234 | 6028 | 365 | 259 | 221 | 5 | 10135 | 10356 |
| 2025 | 183 | 188 | 161 | 1122 | 188 | 195 | 89 | 1296 | 6329 | 383 | 272 | 232 | 5 | 10642 | 10874 |
| 2026 | 192 | 197 | 169 | 1178 | 198 | 205 | 94 | 1361 | 6646 | 402 | 285 | 243 | 5 | 11174 | 11418 |
| 2027 | 201 | 207 | 177 | 1237 | 208 | 216 | 98 | 1429 | 6978 | 422 | 299 | 256 | 5 | 11733 | 11988 |
| 2028 | 211 | 217 | 186 | 1298 | 218 | 226 | 103 | 1500 | 7327 | 443 | 314 | 268 | 6 | 12319 | 12588 |
| 2029 | 222 | 228 | 195 | 1363 | 229 | 238 | 108 | 1575 | 7693 | 465 | 330 | 282 | 6 | 12935 | 13217 |
| 2030 | 233 | 240 | 205 | 1431 | 240 | 249 | 114 | 1654 | 8078 | 489 | 347 | 296 | 6 | 13582 | 13878 |
| 2031 | 245 | 252 | 215 | 1503 | 252 | 262 | 119 | 1737 | 8482 | 513 | 364 | 311 | 7 | 14261 | 14572 |
| 2032 | 257 | 264 | 226 | 1578 | 265 | 275 | 125 | 1823 | 8906 | 539 | 382 | 326 | 7 | 14974 | 15301 |
| 2033 | 270 | 277 | 238 | 1657 | 278 | 289 | 132 | 1915 | 9351 | 566 | 401 | 342 | 7 | 15723 | 16066 |
| 2034 | 283 | 291 | 249 | 1740 | 292 | 303 | 138 | 2010 | 9819 | 594 | 421 | 360 | 8 | 16509 | 16869 |
| 2035 | 298 | 306 | 262 | 1827 | 307 | 318 | 145 | 2111 | 10310 | 624 | 442 | 378 | 8 | 17335 | 17712 |

b) Near H-cross (CH: 27+000 Km)

| Year | Truck |  |  | LCV | Minibus / Van / Tempo | Buses |  | Cars / Jeeps | Motor Cycle | Auto | Tractors | Cycles | Carts | Volume | PCU's |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2-Axle | 3-Axle | Multi-Axle |  |  | Govt Buses | Other Buses |  |  |  |  |  |  |  |  |
| 2015 | 665 | 319 | 205 | 758 | 263 | 199 | 115 | 2054 | 6160 | 488 | 179 | 102 | 13 | 11521 | 12931 |
| 2016 | 699 | 335 | 215 | 796 | 276 | 209 | 121 | 2157 | 6468 | 513 | 188 | 107 | 14 | 12097 | 13578 |
| 2017 | 734 | 352 | 226 | 836 | 290 | 219 | 127 | 2265 | 6791 | 538 | 197 | 112 | 14 | 12702 | 14257 |
| 2018 | 770 | 369 | 237 | 878 | 305 | 230 | 133 | 2378 | 7130 | 565 | 207 | 118 | 15 | 13337 | 14969 |
| 2019 | 809 | 388 | 249 | 922 | 320 | 242 | 140 | 2497 | 7487 | 594 | 217 | 124 | 16 | 14004 | 15718 |
| 2020 | 849 | 407 | 261 | 968 | 336 | 254 | 147 | 2622 | 7861 | 623 | 228 | 130 | 17 | 14704 | 16504 |
| 2021 | 892 | 428 | 274 | 1016 | 353 | 267 | 154 | 2753 | 8254 | 655 | 240 | 136 | 17 | 15439 | 17329 |
| 2022 | 936 | 449 | 288 | 1067 | 370 | 280 | 162 | 2891 | 8667 | 687 | 252 | 143 | 18 | 16211 | 18196 |
| 2023 | 983 | 472 | 302 | 1120 | 389 | 294 | 170 | 3035 | 9100 | 722 | 264 | 150 | 19 | 17021 | 19105 |
| 2024 | 1032 | 495 | 317 | 1176 | 408 | 309 | 179 | 3187 | 9556 | 758 | 277 | 158 | 20 | 17872 | 20061 |
| 2025 | 1084 | 520 | 333 | 1235 | 429 | 324 | 188 | 3346 | 10033 | 796 | 291 | 166 | 21 | 18766 | 21064 |
| 2026 | 1138 | 546 | 350 | 1297 | 450 | 340 | 197 | 3514 | 10535 | 835 | 306 | 174 | 22 | 19704 | 22117 |
| 2027 | 1195 | 573 | 367 | 1362 | 473 | 357 | 207 | 3689 | 11062 | 877 | 321 | 183 | 23 | 20690 | 23223 |
| 2028 | 1255 | 602 | 386 | 1430 | 496 | 375 | 217 | 3874 | 11615 | 921 | 337 | 192 | 25 | 21724 | 24384 |
| 2029 | 1318 | 632 | 405 | 1501 | 521 | 394 | 228 | 4068 | 12196 | 967 | 354 | 201 | 26 | 22810 | 25603 |
| 2030 | 1383 | 663 | 425 | 1576 | 547 | 414 | 239 | 4271 | 12805 | 1015 | 372 | 211 | 27 | 23951 | 26883 |
| 2031 | 1453 | 697 | 447 | 1655 | 574 | 434 | 251 | 4485 | 13446 | 1066 | 390 | 222 | 28 | 25148 | 28227 |
| 2032 | 1525 | 731 | 469 | 1738 | 603 | 456 | 264 | 4709 | 14118 | 1119 | 410 | 233 | 30 | 26406 | 29639 |
| 2033 | 1601 | 768 | 492 | 1825 | 633 | 479 | 277 | 4944 | 14824 | 1175 | 430 | 245 | 31 | 27726 | 31121 |
| 2034 | 1682 | 806 | 517 | 1916 | 665 | 503 | 291 | 5191 | 15565 | 1234 | 452 | 257 | 33 | 29112 | 32677 |
| 2035 | 1766 | 847 | 543 | 2012 | 698 | 528 | 306 | 5451 | 16343 | 1296 | 475 | 270 | 34 | 30568 | 34310 |

c) Near Vemgal (CH: 40+000 Km)

| Year | Truck |  |  | LCV | Minibus / Van / Tempo | Buses |  | Cars / Jeeps | Motor Cycle | Auto | Tractors | Cycles | Carts | Volume | PCU's |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2-Axle | 3-Axle | Multi-Axle |  |  | Govt Buses | Other Buses |  |  |  |  |  |  |  |  |
| 2015 | 210 | 128 | 53 | 408 | 58 | 238 | 75 | 1170 | 4394 | 454 | 68 | 27 | 18 | 7301 | 7172 |
| 2016 | 220 | 135 | 56 | 428 | 61 | 250 | 79 | 1229 | 4614 | 477 | 71 | 28 | 19 | 7666 | 7530 |
| 2017 | 231 | 142 | 58 | 450 | 64 | 262 | 83 | 1290 | 4845 | 501 | 75 | 29 | 20 | 8049 | 7907 |
| 2018 | 243 | 149 | 61 | 472 | 67 | 275 | 87 | 1355 | 5087 | 526 | 79 | 31 | 21 | 8451 | 8302 |
| 2019 | 255 | 156 | 64 | 496 | 71 | 289 | 91 | 1422 | 5341 | 552 | 82 | 32 | 22 | 8874 | 8717 |
| 2020 | 267 | 164 | 68 | 521 | 74 | 304 | 96 | 1493 | 5609 | 580 | 87 | 34 | 23 | 9318 | 9153 |
| 2021 | 281 | 172 | 71 | 547 | 78 | 319 | 100 | 1568 | 5889 | 609 | 91 | 36 | 24 | 9784 | 9611 |
| 2022 | 295 | 181 | 75 | 574 | 82 | 335 | 105 | 1647 | 6183 | 639 | 95 | 38 | 25 | 10273 | 10091 |
| 2023 | 310 | 190 | 78 | 603 | 86 | 351 | 111 | 1729 | 6493 | 671 | 100 | 39 | 26 | 10786 | 10596 |
| 2024 | 325 | 199 | 82 | 633 | 90 | 369 | 116 | 1815 | 6817 | 705 | 105 | 41 | 27 | 11326 | 11125 |
| 2025 | 341 | 209 | 86 | 664 | 95 | 387 | 122 | 1906 | 7158 | 740 | 111 | 44 | 29 | 11892 | 11682 |
| 2026 | 358 | 220 | 91 | 698 | 99 | 407 | 128 | 2001 | 7516 | 777 | 116 | 46 | 30 | 12487 | 12266 |
| 2027 | 376 | 231 | 95 | 732 | 104 | 427 | 134 | 2101 | 7892 | 816 | 122 | 48 | 32 | 13111 | 12879 |
| 2028 | 395 | 242 | 100 | 769 | 110 | 449 | 141 | 2206 | 8286 | 856 | 128 | 50 | 33 | 13767 | 13523 |
| 2029 | 415 | 254 | 105 | 808 | 115 | 471 | 148 | 2317 | 8701 | 899 | 134 | 53 | 35 | 14455 | 14199 |
| 2030 | 436 | 267 | 110 | 848 | 121 | 494 | 156 | 2433 | 9136 | 944 | 141 | 56 | 37 | 15178 | 14909 |
| 2031 | 457 | 280 | 116 | 890 | 127 | 519 | 163 | 2554 | 9592 | 991 | 148 | 58 | 39 | 15937 | 15655 |
| 2032 | 480 | 294 | 121 | 935 | 133 | 545 | 172 | 2682 | 10072 | 1041 | 156 | 61 | 41 | 16733 | 16437 |
| 2033 | 504 | 309 | 128 | 982 | 140 | 572 | 180 | 2816 | 10576 | 1093 | 163 | 64 | 43 | 17570 | 17259 |
| 2034 | 530 | 325 | 134 | 1031 | 147 | 601 | 189 | 2957 | 11105 | 1148 | 171 | 68 | 45 | 18449 | 18122 |
| 2035 | 556 | 341 | 141 | 1082 | 154 | 631 | 199 | 3105 | 11660 | 1205 | 180 | 71 | 47 | 19371 | 19028 |

# CHAPTER 4

ALIGNMENT DESIGN AND PROPOSALS

## 4.1   General

Primary objective of highway design is to make improvements on the project road as per the recommendations provided in the IRC guidelines and special publications of MORT&H. These standards are required to establish the technical, economical, environmental, social, and financial feasibility of the project. As discussed in the previous chapters, the project road passes through plain terrain and hence the appropriate design speed for the respective terrain classification is adopted for the design.

Highway design not only includes the geometric design of road sections, but also the design of intersections, interchanges and provision of wayside amenities and facilities. Design of road sections has been done according to the design standards set for the project and improvements identified. This chapter describes the various features of the horizontal alignment and highlights the design of alignment, intersections and other aspects related to highway appurtenances.

## 4.2   Lane Configuration

As per our Scope of work and traffic survey conducted at location near Moorandahalli

(CH: 52+700 Km) Two Lane has been Proposed.

## 4.2   Geometric Design

## 4.2.1 Base Plan

Base plan of the existing project road showing all natural and manmade features has been prepared using the topographical survey data collected through Total Station Survey. All the features within a band width of 45m have been captured with a unique “description code” during the topographical survey along with the details of existing carriageway centreline, edge of pavement, edge of shoulder, toe line of the embankment etc. This data has been downloaded into the software environment to prepare the base plan. The following activities elucidate the preparation of base plans in more details:

- Download the data into software

- Define main corridor features by joining the points of centreline, edge of pavement, embankment toe line

- Join the points with same description codes for all physical features like rivers, buildings, religious structures, shops, telephone poles, electric poles, cross roads etc within the above specified limits

- Establish break lines for features such as edge of the road, shoulder, nalas, top and bottom of ditches, etc;

- Insert the details of existing cross drainage structures such as bridge number, span arrangement etc.

- Insert details of underground utility services collected from secondary sources.

- Cross check the prepared base plans by “walkover” surveys

- Update and finalize the base plans with additional survey data if necessary

## 4.2.2 Horizontal Alignment Design

The horizontal alignment design for the project road is done using most sophisticated road design software MX Roads. The design standards corresponding to a design speed of 65kmph - 80kmph. Extensive field checks to verify the feasibility of the proposed alignment have been carried out and suitable modifications to the alignment have been done wherever considered essential to safeguard sensitive elements. It can be observed from the base plan that the existing alignment of the project corridor consists of many horizontal curves and few steep vertical curves.

The main criterion that has been followed in finalizing the alignment includes:

- Available ROW from the existing centreline;

- Minimum land acquisition;

- Location of utility lines;

- Abutting land use;

- Adjacent cultural properties;

- Adjacent socially sensitive structures;

- Geometric improvements necessary;

- Location and type of trees required to be cut;

- Transmission line towers; and

- Waterlogged Areas / Ponds.

## 4.2.3 Conceptual Design of Horizontal Alignment

The design standards corresponding to 65kmph and 80kmph have been adopted in general for the project road to ensure uniform and consistent design standards for safety reasons. Locations of horizontal curves have been checked for the adequacy of radius for the set design speed. All the curves satisfy the requirements of design standards for a design speed of 65-80 kmph.

The Alignment of the project road starts at CH: 0+000 Km of MDR at Kolar  and ends at Kolar  @ CH 27+500 Km of MDR. The total length of the project stretch is 27.500 km in Kolar district.

This alignment is designed for the speed of 65 - 80 Kmph throughout the project stretch.

Table 4- 1: Details of Existing Horizontal Curves

| Sl No. | Existing Chainage (Km) |  | Existing Radius of Curve (r) |
| --- | --- | --- | --- |
|  | From | To |  |
| 1 | 1.236 | 1.378 | 341 |
| 2 | 1.6992 | 1.828 | 1121 |
| 3 | 5.069 | 5.219 | 972 |
| 4 | 9.608 | 9.681 | 55 |
| 5 | 9.754 | 9.829 | 100 |
| 6 | 10.306 | 10.364 | 83 |
| 7 | 10.512 | 10.533 | 58 |
| 8 | 10.543 | 10.567 | 46 |
| 9 | 15.038 | 15.085 | 235 |
| 10 | 15.212 | 15.25 | 90 |
| 11 | 15.338 | 15.384 | 193 |
| 12 | 15.415 | 15.444 | 203 |
| 13 | 15.87 | 15.962 | 299 |
| 14 | 16.036 | 16.104 | 101 |
| 15 | 16.116 | 16.175 | 164 |
| 16 | 16.198 | 16.255 | 180 |
| 17 | 16.339 | 16.431 | 252 |
| 18 | 18.792 | 19.038 | 439 |
| 19 | 19.693 | 19.851 | 199 |
| 20 | 23.854 | 24.107 | 953 |
| 21 | 26.074 | 26.206 | 396 |
| 22 | 26.231 | 26.378 | 842 |
| 23 | 31.756 | 32.186 | 1424 |
| 24 | 32.721 | 33.067 | 582 |
| 25 | 33.186 | 33.625 | 595 |
| 26 | 37.02 | 37.233 | 825 |
| 27 | 38.13 | 38.396 | 1158 |
| 28 | 41.53 | 41.776 | 1367 |
| 29 | 43.36 | 43.512 | 666 |
| 30 | 43.512 | 43.689 | 370 |
| 31 | 43.962 | 44.109 | 300 |
| 32 | 44.109 | 44.245 | 688 |
| 33 | 44.971 | 45.399 | 1115 |
| 34 | 49.905 | 50.215 | 1369 |
| 35 | 52.835 | 52.914 | 63 |
| 36 | 53.034 | 53.08 | 116 |
| 37 | 53.082 | 53.195 | 291 |

Table 4- 2: Details of Proposed Horizontal Curves

| Sl. No. | Radius | Transition Length, Ls | Start Chainage of |  | End Chainage of |  | HIP Chainage | Design Speed, V | Δ | Lc | Dc | S | Θ | Es | k | Ts | e | Rotation Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Transition | Curve | Curve | Transition |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 60 | 0 |  | 0+019 | 0+036 |  | 0+027 | 30 | 0.272 | 16.349 | 0.272 | 0.000 | 0.000 | 0.561 | 0.000 | 8.225 | 5.00% | 84.392 |
| 2 | 2200 | 0 |  | 0+728 | 0+752 |  | 0+740 | 100 | 0.011 | 24.129 | 0.011 | 0.000 | 0.000 | 0.033 | 0.000 | 12.065 | NR | 163.349 |
| 3 | 550 | 95 | 1+143 | 1+238 | 1+376 | 1+471 | 1+307 | 100 | 0.423 | 137.915 | 0.251 | 0.684 | 0.086 | 13.263 | 47.488 | 165.865 | 5.00% | 161.841 |
| 4 | 800 | 60 | 1+662 | 1+722 | 1+761 | 1+821 | 1+742 | 100 | 0.124 | 39.34 | 0.049 | 0.188 | 0.038 | 1.732 | 29.999 | 79.744 | 5.00% | 139.409 |
| 5 | 3000 | 0 |  | 2+695 | 2+707 |  | 2+701 | 100 | 0.004 | 12.918 | 0.004 | 0.000 | 0.000 | 0.007 | 0.000 | 6.459 | NR | 131.431 |
| 6 | 3000 | 0 |  | 3+679 | 3+746 |  | 3+713 | 100 | 0.022 | 67.096 | 0.022 | 0.000 | 0.000 | 0.188 | 0.000 | 33.549 | NR | 131.095 |
| 7 | 2200 | 0 |  | 4+085 | 4+143 |  | 4+114 | 100 | 0.027 | 58.597 | 0.027 | 0.000 | 0.000 | 0.195 | 0.000 | 29.300 | NR | 129.814 |
| 8 | 3000 | 0 |  | 4+288 | 4+340 |  | 4+314 | 80 | 0.017 | 51.758 | 0.017 | 0.000 | 0.000 | 0.112 | 0.000 | 25.880 | NR | 131.340 |
| 9 | 800 | 30 | 4+964 | 4+994 | 5+025 | 5+055 | 5+009 | 80 | 0.076 | 30.58 | 0.038 | 0.047 | 0.019 | 0.621 | 15.000 | 45.306 | 3.56% | 130.390 |
| 10 | 500 | 45 | 5+099 | 5+144 | 5+186 | 5+231 | 5+165 | 80 | 0.173 | 41.743 | 0.083 | 0.169 | 0.045 | 2.056 | 22.498 | 65.994 | 5.00% | 133.922 |
| 11 | 500 | 45 | 5+331 | 5+376 | 5+427 | 5+472 | 5+402 | 80 | 0.191 | 50.363 | 0.101 | 0.169 | 0.045 | 2.452 | 22.498 | 70.341 | 5.00% | 125.579 |
| 12 | 3000 | 0 |  | 5+852 | 5+913 |  | 5+882 | 100 | 0.020 | 61.029 | 0.020 | 0.000 | 0.000 | 0.155 | 0.000 | 30.516 | NR | 135.462 |
| 13 | 3000 | 0 |  | 5+963 | 6+026 |  | 5+994 | 100 | 0.021 | 62.759 | 0.021 | 0.000 | 0.000 | 0.164 | 0.000 | 31.381 | NR | 134.512 |
| 14 | 3000 | 0 |  | 7+995 | 7+995 |  | 7+995 | 100 | 0.000 | 0.174 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.087 | NR | 135.711 |
| 15 | 3000 | 0 |  | 9+072 | 9+103 |  | 9+088 | 100 | 0.010 | 30.106 | 0.010 | 0.000 | 0.000 | 0.038 | 0.000 | 15.053 | NR | 135.714 |
| 16 | 200 | 60 | 9+351 | 9+411 | 9+505 | 9+565 | 9+458 | 65 | 0.771 | 94.202 | 0.471 | 0.750 | 0.150 | 16.650 | 29.978 | 111.444 | 5.00% | 134.717 |
| 17 | 500 | 25 | 10+009 | 10+034 | 10+088 | 10+113 | 10+061 | 65 | 0.158 | 53.972 | 0.108 | 0.052 | 0.025 | 1.615 | 12.500 | 52.072 | 3.76% | 92.994 |
| 18 | 300 | 75 | 10+149 | 10+224 | 10+450 | 10+525 | 10+337 | 80 | 1.006 | 226.653 | 0.756 | 0.781 | 0.125 | 43.256 | 37.480 | 202.876 | 5.00% | 86.909 |
| 19 | 2000 | 0 |  | 10+668 | 10+700 |  | 10+684 | 100 | 0.016 | 32.396 | 0.016 | 0.000 | 0.000 | 0.066 | 0.000 | 16.198 | NR | 137.753 |
| 20 | 3000 | 0 |  | 11+103 | 11+113 |  | 11+108 | 100 | 0.003 | 9.67 | 0.003 | 0.000 | 0.000 | 0.004 | 0.000 | 4.835 | NR | 139.747 |
| 21 | 3000 | 0 |  | 11+440 | 11+495 |  | 11+467 | 100 | 0.019 | 55.534 | 0.019 | 0.000 | 0.000 | 0.129 | 0.000 | 27.767 | NR | 139.932 |
| 22 | 3000 | 0 |  | 11+769 | 11+824 |  | 11+796 | 100 | 0.018 | 54.194 | 0.018 | 0.000 | 0.000 | 0.122 | 0.000 | 27.098 | NR | 138.871 |
| 23 | 3000 | 0 |  | 13+078 | 13+083 |  | 13+080 | 100 | 0.002 | 4.799 | 0.002 | 0.000 | 0.000 | 0.001 | 0.000 | 2.400 | NR | 139.906 |
| 24 | 3000 | 0 |  | 14+122 | 14+148 |  | 14+135 | 100 | 0.009 | 26.217 | 0.009 | 0.000 | 0.000 | 0.029 | 0.000 | 13.109 | NR | 139.998 |
| 25 | 2200 | 0 |  | 14+425 | 14+447 |  | 14+436 | 100 | 0.010 | 22.625 | 0.010 | 0.000 | 0.000 | 0.029 | 0.000 | 11.313 | NR | 140.499 |
| 26 | 1500 | 35 | 14+498 | 14+533 | 14+603 | 14+638 | 14+568 | 100 | 0.070 | 70.012 | 0.047 | 0.034 | 0.012 | 0.954 | 17.500 | 70.029 | 2.96% | 140.145 |
| 27 | 2200 | 0 |  | 14+708 | 14+745 |  | 14+726 | 100 | 0.017 | 36.891 | 0.017 | 0.000 | 0.000 | 0.077 | 0.000 | 18.446 | NR | 143.723 |
| 28 | 400 | 55 | 14+905 | 14+960 | 15+196 | 15+251 | 15+078 | 85 | 0.727 | 235.658 | 0.589 | 0.315 | 0.069 | 28.272 | 27.496 | 179.696 | 7.00% | 144.509 |
| 29 | 360 | 60 | 15+265 | 15+325 | 15+380 | 15+440 | 15+353 | 100 | 0.320 | 55.352 | 0.154 | 0.417 | 0.083 | 5.092 | 29.993 | 88.235 | 7.00% | 1.796 |
| 30 | 2200 | 0 |  | 15+550 | 15+565 |  | 15+558 | 100 | 0.007 | 14.652 | 0.007 | 0.000 | 0.000 | 0.012 | 0.000 | 7.326 | NR | 167.345 |
| 31 | 1200 | 0 |  | 15+676 | 15+797 |  | 15+737 | 80 | 0.101 | 120.652 | 0.101 | 0.000 | 0.000 | 1.518 | 0.000 | 60.377 | NR | 165.853 |
| 32 | 360 | 60 | 15+879 | 15+939 | 16+124 | 16+184 | 16+032 | 80 | 0.680 | 184.869 | 0.514 | 0.417 | 0.083 | 22.315 | 29.993 | 157.524 | 7.00% | 169.774 |
| 33 | 360 | 60 | 16+197 | 16+257 | 16+394 | 16+454 | 16+326 | 80 | 0.550 | 137.893 | 0.383 | 0.417 | 0.083 | 14.472 | 29.993 | 131.626 | 7.00% | 135.928 |
| 34 | 2200 | 0 |  | 16+731 | 16+763 |  | 16+747 | 100 | 0.015 | 32.76 | 0.015 | 0.000 | 0.000 | 0.061 | 0.000 | 16.380 | NR | 163.122 |
| 35 | 2200 | 0 |  | 16+937 | 16+944 |  | 16+940 | 100 | 0.003 | 6.941 | 0.003 | 0.000 | 0.000 | 0.003 | 0.000 | 3.471 | NR | 163.284 |
| 36 | 2200 | 0 |  | 17+122 | 17+143 |  | 17+133 | 100 | 0.009 | 20.269 | 0.009 | 0.000 | 0.000 | 0.023 | 0.000 | 10.134 | NR | 163.465 |
| 37 | 2200 | 0 |  | 17+483 | 17+502 |  | 17+492 | 100 | 0.009 | 18.989 | 0.009 | 0.000 | 0.000 | 0.020 | 0.000 | 9.494 | NR | 163.993 |
| 38 | 2200 | 0 |  | 18+192 | 18+205 |  | 18+198 | 100 | 0.006 | 12.784 | 0.006 | 0.000 | 0.000 | 0.009 | 0.000 | 6.392 | NR | 163.498 |
| 39 | 2200 | 0 |  | 18+505 | 18+542 |  | 18+523 | 100 | 0.017 | 37.119 | 0.017 | 0.000 | 0.000 | 0.078 | 0.000 | 18.560 | NR | 163.831 |
| 40 | 300 | 75 | 18+705 | 18+780 | 18+866 | 18+941 | 18+823 | 80 | 0.536 | 85.751 | 0.286 | 0.781 | 0.125 | 11.909 | 37.480 | 120.050 | 7.00% | 161.237 |
| 41 | 300 | 75 | 19+536 | 19+611 | 19+728 | 19+803 | 19+669 | 80 | 0.639 | 116.746 | 0.389 | 0.781 | 0.125 | 16.822 | 37.480 | 137.015 | 5.00% | 133.449 |
| 42 | 2200 | 0 |  | 19+935 | 19+960 |  | 19+948 | 100 | 0.012 | 25.628 | 0.012 | 0.000 | 0.000 | 0.037 | 0.000 | 12.814 | NR | 166.625 |
| 43 | 2200 | 0 |  | 20+222 | 20+263 |  | 20+243 | 100 | 0.019 | 40.915 | 0.019 | 0.000 | 0.000 | 0.095 | 0.000 | 20.458 | NR | 169.452 |
| 44 | 2200 | 0 |  | 20+391 | 20+433 |  | 20+412 | 100 | 0.019 | 41.875 | 0.019 | 0.000 | 0.000 | 0.100 | 0.000 | 20.938 | NR | 168.386 |
| 45 | 2200 | 0 |  | 20+696 | 20+702 |  | 20+699 | 100 | 0.002 | 5.225 | 0.002 | 0.000 | 0.000 | 0.002 | 0.000 | 2.613 | NR | 169.477 |
| 46 | 2500 | 0 |  | 20+875 | 20+882 |  | 20+878 | 100 | 0.003 | 7.679 | 0.003 | 0.000 | 0.000 | 0.003 | 0.000 | 3.840 | NR | 169.341 |
| 47 | 2200 | 0 |  | 21+075 | 21+102 |  | 21+088 | 100 | 0.012 | 26.728 | 0.012 | 0.000 | 0.000 | 0.041 | 0.000 | 13.364 | NR | 169.516 |
| 48 | 2200 | 0 |  | 21+378 | 21+398 |  | 21+388 | 100 | 0.009 | 20.143 | 0.009 | 0.000 | 0.000 | 0.023 | 0.000 | 10.072 | NR | 168.821 |
| 49 | 2200 | 0 |  | 21+575 | 21+601 |  | 21+588 | 100 | 0.012 | 26.187 | 0.012 | 0.000 | 0.000 | 0.039 | 0.000 | 13.093 | NR | 169.345 |
| 50 | 2200 | 0 |  | 22+012 | 22+023 |  | 22+018 | 100 | 0.005 | 11.093 | 0.005 | 0.000 | 0.000 | 0.007 | 0.000 | 5.546 | NR | 168.663 |
| 51 | 2200 | 0 |  | 22+335 | 22+382 |  | 22+358 | 100 | 0.021 | 47.199 | 0.021 | 0.000 | 0.000 | 0.127 | 0.000 | 23.600 | NR | 168.952 |
| 52 | 2200 | 0 |  | 22+412 | 22+496 |  | 22+454 | 80 | 0.038 | 84.262 | 0.038 | 0.000 | 0.000 | 0.403 | 0.000 | 42.136 | NR | 170.181 |
| 53 | 2200 | 0 |  | 22+533 | 22+560 |  | 22+547 | 100 | 0.012 | 26.983 | 0.012 | 0.000 | 0.000 | 0.041 | 0.000 | 13.492 | NR | 167.986 |
| 54 | 2200 | 0 |  | 22+806 | 22+814 |  | 22+810 | 100 | 0.004 | 8.04 | 0.004 | 0.000 | 0.000 | 0.004 | 0.000 | 4.021 | NR | 168.690 |
| 55 | 2200 | 0 |  | 23+126 | 23+133 |  | 23+130 | 100 | 0.003 | 7.492 | 0.003 | 0.000 | 0.000 | 0.003 | 0.000 | 3.746 | NR | 168.899 |
| 56 | 2200 | 0 |  | 23+501 | 23+505 |  | 23+503 | 100 | 0.002 | 4.685 | 0.002 | 0.000 | 0.000 | 0.001 | 0.000 | 2.343 | NR | 169.094 |
| 57 | 450 | 50 | 23+806 | 23+856 | 23+916 | 23+966 | 23+886 | 80 | 0.245 | 60.166 | 0.134 | 0.231 | 0.056 | 3.626 | 24.997 | 80.386 | 6.32% | 168.583 |
| 58 | 2200 | 0 |  | 24+128 | 24+138 |  | 24+133 | 100 | 0.004 | 9.345 | 0.004 | 0.000 | 0.000 | 0.005 | 0.000 | 4.673 | NR | 155.548 |
| 59 | 2200 | 0 |  | 24+300 | 24+321 |  | 24+310 | 100 | 0.010 | 21.475 | 0.010 | 0.000 | 0.000 | 0.026 | 0.000 | 10.737 | NR | 154.702 |
| 60 | 2200 | 0 |  | 24+628 | 24+656 |  | 24+642 | 100 | 0.013 | 28.547 | 0.013 | 0.000 | 0.000 | 0.046 | 0.000 | 14.274 | NR | 155.261 |
| 61 | 2200 | 0 |  | 24+934 | 24+946 |  | 24+940 | 100 | 0.005 | 11.581 | 0.005 | 0.000 | 0.000 | 0.008 | 0.000 | 5.790 | NR | 154.518 |
| 62 | 2200 | 0 |  | 25+172 | 25+186 |  | 25+179 | 100 | 0.007 | 14.873 | 0.007 | 0.000 | 0.000 | 0.013 | 0.000 | 7.436 | NR | 154.819 |
| 63 | 2200 | 0 |  | 25+389 | 25+399 |  | 25+394 | 100 | 0.005 | 9.914 | 0.005 | 0.000 | 0.000 | 0.006 | 0.000 | 4.957 | NR | 155.207 |
| 64 | 2200 | 0 |  | 25+545 | 25+565 |  | 25+555 | 100 | 0.009 | 20.45 | 0.009 | 0.000 | 0.000 | 0.024 | 0.000 | 10.225 | NR | 154.948 |
| 65 | 2200 | 0 |  | 26+045 | 26+054 |  | 26+050 | 100 | 0.004 | 8.818 | 0.004 | 0.000 | 0.000 | 0.004 | 0.000 | 4.409 | NR | 154.416 |
| 66 | 2200 | 0 |  | 26+408 | 26+412 |  | 26+410 | 100 | 0.002 | 4.205 | 0.002 | 0.000 | 0.000 | 0.001 | 0.000 | 2.102 | NR | 154.645 |
| 67 | 2200 | 0 |  | 27+382 | 27+395 |  | 27+388 | 100 | 0.006 | 12.711 | 0.006 | 0.000 | 0.000 | 0.009 | 0.000 | 6.356 | NR | 154.536 |
| 68 | 2200 | 0 |  | 28+288 | 28+300 |  | 28+294 | 100 | 0.005 | 11.383 | 0.005 | 0.000 | 0.000 | 0.007 | 0.000 | 5.691 | NR | 154.205 |
| 69 | 2200 | 0 |  | 28+634 | 28+637 |  | 28+636 | 100 | 0.001 | 2.979 | 0.001 | 0.000 | 0.000 | 0.001 | 0.000 | 1.489 | NR | 153.908 |
| 70 | 1200 | 0 |  | 29+021 | 29+026 |  | 29+024 | 100 | 0.004 | 4.888 | 0.004 | 0.000 | 0.000 | 0.002 | 0.000 | 2.444 | 3.70% | 153.986 |
| 71 | 2200 | 0 |  | 29+584 | 29+634 |  | 29+609 | 100 | 0.023 | 50.653 | 0.023 | 0.000 | 0.000 | 0.146 | 0.000 | 25.328 | NR | 153.753 |
| 72 | 2200 | 0 |  | 30+125 | 30+131 |  | 30+128 | 100 | 0.002 | 5.209 | 0.002 | 0.000 | 0.000 | 0.002 | 0.000 | 2.605 | NR | 152.433 |
| 73 | 2200 | 0 |  | 31+197 | 31+207 |  | 31+202 | 100 | 0.005 | 10.16 | 0.005 | 0.000 | 0.000 | 0.006 | 0.000 | 5.080 | NR | 152.298 |
| 74 | 2200 | 0 |  | 31+342 | 31+372 |  | 31+357 | 100 | 0.013 | 29.392 | 0.013 | 0.000 | 0.000 | 0.049 | 0.000 | 14.696 | NR | 152.563 |
| 75 | 600 | 80 | 31+704 | 31+784 | 31+820 | 31+900 | 31+802 | 100 | 0.193 | 35.844 | 0.060 | 0.444 | 0.067 | 3.253 | 39.994 | 98.140 | 7.00% | 151.414 |
| 76 | 2200 | 0 |  | 32+244 | 32+261 |  | 32+253 | 100 | 0.008 | 17.186 | 0.008 | 0.000 | 0.000 | 0.017 | 0.000 | 8.593 | NR | 141.112 |
| 77 | 2200 | 0 |  | 32+517 | 32+535 |  | 32+526 | 100 | 0.008 | 17.845 | 0.008 | 0.000 | 0.000 | 0.018 | 0.000 | 8.922 | NR | 141.182 |
| 78 | 2200 | 0 |  | 32+561 | 32+620 |  | 32+591 | 100 | 0.027 | 59.002 | 0.027 | 0.000 | 0.000 | 0.198 | 0.000 | 29.503 | NR | 140.717 |
| 79 | 450 | 55 | 32+651 | 32+706 | 32+813 | 32+868 | 32+760 | 80 | 0.359 | 106.739 | 0.237 | 0.280 | 0.061 | 7.650 | 27.497 | 109.299 | 6.32% | 137.692 |
| 80 | 2200 | 0 |  | 33+012 | 33+029 |  | 33+020 | 100 | 0.008 | 16.605 | 0.008 | 0.000 | 0.000 | 0.016 | 0.000 | 8.303 | NR | 119.553 |
| 81 | 400 | 55 | 33+198 | 33+253 | 33+397 | 33+452 | 33+325 | 80 | 0.499 | 144.464 | 0.361 | 0.315 | 0.069 | 13.089 | 27.496 | 129.427 | 7.00% | 119.334 |
| 82 | 2200 | 0 |  | 34+341 | 34+351 |  | 34+346 | 100 | 0.005 | 10.353 | 0.005 | 0.000 | 0.000 | 0.006 | 0.000 | 5.177 | NR | 146.374 |
| 83 | 2200 | 0 |  | 34+833 | 34+839 |  | 34+836 | 100 | 0.003 | 5.807 | 0.003 | 0.000 | 0.000 | 0.002 | 0.000 | 2.904 | NR | 146.996 |
| 84 | 2200 | 0 |  | 35+693 | 35+698 |  | 35+696 | 100 | 0.002 | 5.022 | 0.002 | 0.000 | 0.000 | 0.001 | 0.000 | 2.511 | NR | 146.845 |
| 85 | 2200 | 0 |  | 35+925 | 35+939 |  | 35+932 | 100 | 0.006 | 13.734 | 0.006 | 0.000 | 0.000 | 0.011 | 0.000 | 6.867 | NR | 146.975 |
| 86 | 2200 | 0 |  | 36+422 | 36+425 |  | 36+424 | 100 | 0.001 | 3.105 | 0.001 | 0.000 | 0.000 | 0.001 | 0.000 | 1.552 | NR | 146.618 |
| 87 | 2200 | 0 |  | 36+893 | 36+903 |  | 36+898 | 100 | 0.005 | 10.164 | 0.005 | 0.000 | 0.000 | 0.006 | 0.000 | 5.082 | NR | 146.699 |
| 88 | 600 | 80 | 36+947 | 37+027 | 37+047 | 37+127 | 37+037 | 100 | 0.167 | 20.368 | 0.034 | 0.444 | 0.067 | 2.551 | 39.994 | 90.333 | 7.00% | 147.975 |
| 89 | 2200 | 0 |  | 37+214 | 37+216 |  | 37+215 | 100 | 0.001 | 2.056 | 0.001 | 0.000 | 0.000 | 0.000 | 0.000 | 1.029 | NR | 155.758 |
| 90 | 2200 | 0 |  | 37+843 | 37+865 |  | 37+854 | 100 | 0.010 | 22.33 | 0.010 | 0.000 | 0.000 | 0.028 | 0.000 | 11.165 | NR | 156.601 |
| 91 | 2100 | 0 |  | 37+958 | 38+262 |  | 38+110 | 100 | 0.145 | 304.198 | 0.145 | 0.000 | 0.000 | 5.520 | 0.000 | 152.365 | NR | 156.020 |
| 92 | 2200 | 0 |  | 38+762 | 38+774 |  | 38+768 | 100 | 0.005 | 11.833 | 0.005 | 0.000 | 0.000 | 0.008 | 0.000 | 5.917 | NR | 147.720 |
| 93 | 2200 | 0 |  | 39+318 | 39+320 |  | 39+319 | 100 | 0.001 | 1.866 | 0.001 | 0.000 | 0.000 | 0.000 | 0.000 | 0.933 | NR | 148.028 |
| 94 | 2200 | 0 |  | 39+715 | 39+723 |  | 39+719 | 100 | 0.004 | 8.111 | 0.004 | 0.000 | 0.000 | 0.004 | 0.000 | 4.056 | NR | 148.077 |
| 95 | 2200 | 0 |  | 40+168 | 40+179 |  | 40+173 | 100 | 0.005 | 11.443 | 0.005 | 0.000 | 0.000 | 0.007 | 0.000 | 5.721 | NR | 147.866 |
| 96 | 2200 | 0 |  | 41+294 | 41+303 |  | 41+298 | 100 | 0.004 | 9.194 | 0.004 | 0.000 | 0.000 | 0.005 | 0.000 | 4.597 | NR | 147.568 |
| 97 | 700 | 70 | 41+532 | 41+602 | 41+681 | 41+751 | 41+642 | 100 | 0.212 | 78.634 | 0.112 | 0.292 | 0.050 | 4.257 | 34.997 | 109.626 | 6.35% | 148.330 |
| 98 | 1200 | 0 |  | 42+406 | 42+406 |  | 42+406 | 100 | 0.000 | 0.441 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.220 | 3.70% | 159.738 |
| 99 | 2200 | 0 |  | 43+087 | 43+092 |  | 43+089 | 100 | 0.003 | 5.687 | 0.003 | 0.000 | 0.000 | 0.002 | 0.000 | 2.844 | NR | 159.952 |
| 100 | 360 | 60 | 43+252 | 43+312 | 43+319 | 43+379 | 43+315 | 80 | 0.186 | 6.9 | 0.019 | 0.417 | 0.083 | 1.978 | 29.993 | 63.578 | 7.00% | 159.604 |
| 101 | 300 | 75 | 43+395 | 43+470 | 43+515 | 43+590 | 43+493 | 80 | 0.401 | 45.273 | 0.151 | 0.781 | 0.125 | 6.927 | 37.480 | 98.594 | 7.00% | 152.008 |
| 102 | 300 | 75 | 43+857 | 43+932 | 43+949 | 44+024 | 43+940 | 80 | 0.307 | 16.957 | 0.057 | 0.781 | 0.125 | 4.349 | 37.480 | 83.943 | 7.00% | 172.196 |
| 103 | 500 | 45 | 44+033 | 44+078 | 44+081 | 44+126 | 44+079 | 80 | 0.096 | 3.044 | 0.006 | 0.169 | 0.045 | 0.747 | 22.498 | 46.547 | 5.69% | 7.951 |
| 104 | 2200 | 0 |  | 44+517 | 44+749 |  | 44+633 | 100 | 0.106 | 232.258 | 0.106 | 0.000 | 0.000 | 3.069 | 0.000 | 116.237 | NR | 4.557 |
| 105 | 2500 | 0 |  | 44+840 | 45+006 |  | 44+923 | 100 | 0.066 | 166.097 | 0.066 | 0.000 | 0.000 | 1.380 | 0.000 | 83.079 | NR | 178.431 |
| 106 | 500 | 45 | 45+068 | 45+113 | 45+199 | 45+244 | 45+156 | 80 | 0.262 | 85.92 | 0.172 | 0.169 | 0.045 | 4.486 | 22.498 | 88.357 | 5.69% | 173.984 |
| 107 | 2200 | 0 |  | 45+312 | 45+335 |  | 45+323 | 100 | 0.010 | 22.942 | 0.010 | 0.000 | 0.000 | 0.030 | 0.000 | 11.471 | NR | 160.519 |
| 108 | 2200 | 0 |  | 45+406 | 45+437 |  | 45+422 | 100 | 0.014 | 31.564 | 0.014 | 0.000 | 0.000 | 0.057 | 0.000 | 15.782 | NR | 159.025 |
| 109 | 2200 | 0 |  | 45+671 | 45+681 |  | 45+676 | 100 | 0.005 | 9.958 | 0.005 | 0.000 | 0.000 | 0.006 | 0.000 | 4.979 | NR | 159.847 |
| 110 | 2200 | 0 |  | 45+889 | 45+911 |  | 45+900 | 100 | 0.010 | 21.377 | 0.010 | 0.000 | 0.000 | 0.026 | 0.000 | 10.689 | NR | 160.107 |
| 111 | 2200 | 0 |  | 46+789 | 46+802 |  | 46+795 | 100 | 0.006 | 12.802 | 0.006 | 0.000 | 0.000 | 0.009 | 0.000 | 6.401 | NR | 159.550 |
| 112 | 2200 | 0 |  | 46+948 | 46+977 |  | 46+962 | 100 | 0.013 | 28.24 | 0.013 | 0.000 | 0.000 | 0.045 | 0.000 | 14.120 | NR | 159.883 |
| 113 | 2200 | 0 |  | 47+126 | 47+159 |  | 47+142 | 100 | 0.015 | 33.129 | 0.015 | 0.000 | 0.000 | 0.062 | 0.000 | 16.565 | NR | 159.148 |
| 114 | 2200 | 0 |  | 47+449 | 47+495 |  | 47+472 | 100 | 0.021 | 46.033 | 0.021 | 0.000 | 0.000 | 0.120 | 0.000 | 23.017 | NR | 160.010 |
| 115 | 2200 | 0 |  | 47+572 | 47+596 |  | 47+584 | 100 | 0.011 | 23.333 | 0.011 | 0.000 | 0.000 | 0.031 | 0.000 | 11.667 | NR | 158.812 |
| 116 | 2200 | 0 |  | 48+154 | 48+165 |  | 48+160 | 100 | 0.005 | 11.405 | 0.005 | 0.000 | 0.000 | 0.007 | 0.000 | 5.702 | NR | 159.419 |
| 117 | 2200 | 0 |  | 48+400 | 48+405 |  | 48+403 | 100 | 0.002 | 4.958 | 0.002 | 0.000 | 0.000 | 0.001 | 0.000 | 2.479 | NR | 159.716 |
| 118 | 2200 | 0 |  | 48+895 | 48+912 |  | 48+903 | 100 | 0.008 | 16.771 | 0.008 | 0.000 | 0.000 | 0.016 | 0.000 | 8.385 | NR | 159.587 |
| 119 | 2200 | 0 |  | 49+007 | 49+077 |  | 49+042 | 100 | 0.032 | 70.239 | 0.032 | 0.000 | 0.000 | 0.280 | 0.000 | 35.123 | NR | 159.150 |
| 120 | 2200 | 0 |  | 49+118 | 49+188 |  | 49+153 | 100 | 0.032 | 69.888 | 0.032 | 0.000 | 0.000 | 0.278 | 0.000 | 34.947 | NR | 160.980 |
| 121 | 2200 | 0 |  | 49+303 | 49+318 |  | 49+310 | 100 | 0.007 | 15.116 | 0.007 | 0.000 | 0.000 | 0.013 | 0.000 | 7.558 | NR | 159.159 |
| 122 | 2200 | 0 |  | 49+443 | 49+479 |  | 49+461 | 100 | 0.016 | 35.929 | 0.016 | 0.000 | 0.000 | 0.073 | 0.000 | 17.964 | NR | 158.766 |
| 123 | 400 | 55 | 49+845 | 49+900 | 49+932 | 49+987 | 49+916 | 80 | 0.216 | 31.31 | 0.078 | 0.315 | 0.069 | 2.656 | 27.496 | 70.853 | 7.00% | 159.407 |
| 124 | 2200 | 0 |  | 50+372 | 50+395 |  | 50+384 | 100 | 0.010 | 23.041 | 0.010 | 0.000 | 0.000 | 0.030 | 0.000 | 11.520 | NR | 147.625 |
| 125 | 2200 | 0 |  | 50+602 | 50+625 |  | 50+614 | 100 | 0.011 | 23.391 | 0.011 | 0.000 | 0.000 | 0.031 | 0.000 | 11.695 | NR | 147.938 |
| 126 | 2200 | 0 |  | 51+293 | 51+303 |  | 51+298 | 100 | 0.005 | 10.353 | 0.005 | 0.000 | 0.000 | 0.006 | 0.000 | 5.177 | NR | 147.329 |
| 127 | 2200 | 0 |  | 51+391 | 51+406 |  | 51+399 | 100 | 0.006 | 14.225 | 0.006 | 0.000 | 0.000 | 0.011 | 0.000 | 7.113 | NR | 147.599 |
| 128 | 2200 | 0 |  | 51+788 | 51+808 |  | 51+798 | 100 | 0.009 | 19.951 | 0.009 | 0.000 | 0.000 | 0.023 | 0.000 | 9.976 | NR | 147.229 |
| 129 | 2200 | 0 |  | 51+864 | 51+944 |  | 51+904 | 100 | 0.037 | 80.45 | 0.037 | 0.000 | 0.000 | 0.368 | 0.000 | 40.229 | NR | 146.709 |
| 130 | 2200 | 0 |  | 52+027 | 52+081 |  | 52+054 | 100 | 0.025 | 54.02 | 0.025 | 0.000 | 0.000 | 0.166 | 0.000 | 27.012 | NR | 148.804 |
| 131 | 2200 | 0 |  | 52+196 | 52+223 |  | 52+209 | 100 | 0.012 | 27.261 | 0.012 | 0.000 | 0.000 | 0.042 | 0.000 | 13.630 | NR | 147.397 |
| 132 | 2200 | 0 |  | 52+433 | 52+457 |  | 52+445 | 100 | 0.011 | 23.291 | 0.011 | 0.000 | 0.000 | 0.031 | 0.000 | 11.646 | NR | 148.107 |
| 133 | 1200 | 0 |  | 52+650 | 52+670 |  | 52+660 | 100 | 0.017 | 20.154 | 0.017 | 0.000 | 0.000 | 0.042 | 0.000 | 10.077 | 3.70% | 147.501 |
| 134 | 1200 | 0 |  | 52+678 | 52+720 |  | 52+699 | 100 | 0.035 | 41.733 | 0.035 | 0.000 | 0.000 | 0.181 | 0.000 | 20.869 | 3.70% | 146.537 |
| 135 | 38 | 0 |  | 52+734 | 52+780 |  | 52+757 | 20 | 1.210 | 45.961 | 1.210 | 0.000 | 0.000 | 8.193 | 0.000 | 26.263 | 4.68% | 148.532 |
| 136 | 2200 | 0 |  | 52+817 | 52+832 |  | 52+824 | 100 | 0.007 | 14.421 | 0.007 | 0.000 | 0.000 | 0.012 | 0.000 | 7.211 | NR | 79.231 |
| 137 | 60 | 55 | 52+878 | 52+933 | 52+956 | 53+011 | 52+945 | 30 | 1.294 | 22.652 | 0.378 | 2.101 | 0.458 | 17.836 | 27.309 | 74.234 | 5.00% | 78.856 |
| 138 | 45 | 70 | 52+938 | 53+008 | 53+021 | 53+091 | 53+015 | 30 | 1.858 | 13.607 | 0.302 | 4.537 | 0.778 | 37.746 | 34.306 | 100.586 | 5.00% | 100.487 |
| 139 | 1500 | 35 | 53+120 | 53+155 | 53+209 | 53+244 | 53+182 | 100 | 0.059 | 53.762 | 0.036 | 0.034 | 0.012 | 0.691 | 17.500 | 61.895 | 2.96% | 83.162 |
| 140 | 1300 | 40 | 53+379 | 53+419 | 53+488 | 53+528 | 53+453 | 100 | 0.084 | 68.604 | 0.053 | 0.051 | 0.015 | 1.186 | 20.000 | 74.336 | 3.42% | 81.109 |

## 4.3   Right of Way

As per IRC standards, the RoW should normally be 45m, but can vary in the range of 30m to 60m. The land width requirement for the Project road shall be assessed depending upon the extent of the existing RoW and the proposed improvements. The minimum berm width required between the Toe of embankment to RoW boundary is 5m keeping in view the width of longitudinal drain at the outer end of RoW as 2m. This is required to accommodate tree plantation and for the vehicle driver to bring the vehicle under control in the event of swerving off and leaving the roadway.

Table 4- 3: Existing ROW details (as per Inventory)

| Sl No | Chainage (km) |  | Length (Km) | Row (m) (Approx) | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | From | To |  |  |  |
| 1 | 0.000 | 1.000 | 1 | 15 | Nandi cross Doddamarali |
| 2 | 1.000 | 2.000 | 1 | 15 | - |
| 3 | 2.000 | 3.000 | 1 | 15 | - |
| 4 | 3.000 | 4.000 | 1 | 15 | - |
| 5 | 4.000 | 5.000 | 1 | 12 | - |
| 6 | 5.000 | 6.000 | 1 | 12 | Narayanapura |
| 7 | 6.000 | 7.000 | 1 | 15 | - |
| 8 | 7.000 | 8.000 | 1 | 15 | - |
| 9 | 8.000 | 9.000 | 1 | 12 | Rodahalli |
| 10 | 9.000 | 10.000 | 1 | 12 | Chikkanahalli |
| 11 | 10.000 | 11.000 | 1 | 7 | - |
| 12 | 11.000 | 12.000 | 1 | 10 | Vijayapura |
| 13 | 12.000 | 13.000 | 1 | 22 | - |
| 14 | 13.000 | 14.000 | 1 | 20 | - |
| 15 | 14.000 | 15.000 | 1 | 20 | - |
| 16 | 15.000 | 16.000 | 1 | 25 | - |
| 17 | 16.000 | 17.000 | 1 | 12 | - |
| 18 | 17.000 | 18.000 | 1 | 18 | - |
| 19 | 18.000 | 19.000 | 1 | 20 | - |
| 20 | 19.000 | 20.000 | 1 | 22 | - |
| 21 | 20.000 | 21.000 | 1 | 15 | - |
| 22 | 21.000 | 22.000 | 1 | 18 | - |
| 23 | 22.000 | 23.000 | 1 | 20 | - |
| 24 | 23.000 | 24.000 | 1 | 20 | - |
| 25 | 24.000 | 25.000 | 1 | 20 | - |
| 26 | 25.000 | 26.000 | 1 | 30 | - |
| 27 | 26.000 | 27.000 | 1 | 20 | - |
| 28 | 27.000 | 28.000 | 1 | 20 | - |
| 29 | 28.000 | 29.000 | 1 | 20 | - |
| 30 | 29.000 | 30.000 | 1 | 20 | - |
| 31 | 30.000 | 31.000 | 1 | 25 | - |
| 32 | 31.000 | 32.000 | 1 | 20 | - |
| 33 | 32.000 | 33.000 | 1 | 16 | - |
| 34 | 33.000 | 34.000 | 1 | 15 | - |
| 35 | 34.000 | 35.000 | 1 | 20 | - |
| 36 | 35.000 | 36.000 | 1 | 20 | - |
| 37 | 36.000 | 37.000 | 1 | 15 | - |
| 38 | 37.000 | 38.000 | 1 | 10 | - |
| 39 | 38.000 | 39.000 | 1 | 15 | - |
| 40 | 39.000 | 40.000 | 1 | 15 | - |
| 41 | 40.000 | 41.000 | 1 | 15 | - |
| 42 | 41.000 | 42.000 | 1 | 15 | - |
| 43 | 42.000 | 43.000 | 1 | 15 | - |
| 44 | 43.000 | 44.000 | 1 | 22 | - |
| 45 | 44.000 | 45.000 | 1 | 17 | - |
| 46 | 45.000 | 46.000 | 1 | 16 | - |
| 47 | 46.000 | 47.000 | 1 | 20 | - |
| 48 | 47.000 | 48.000 | 1 | 15 | - |
| 49 | 48.000 | 49.000 | 1 | 15 | - |
| 50 | 49.000 | 50.000 | 1 | 15 | - |
| 51 | 50.000 | 51.000 | 1 | 20 | - |
| 52 | 51.000 | 52.000 | 1 | 20 | - |
| 53 | 52.000 | 53.000 | 1 | 15 | - |
| 54 | 53.000 | 54.000 | 1 | 15 | - |

## 4.4   Existing Intersections

There are many intersections along with cross roads having bituminous surfacing and falling into the different categories of Rural roads, ODR, MDR and SH, most of them being rural roads. The project road provides direct access to many villages.

Based on turning moments, detailed designs shall be carried during Detailed Project Report preparation, keeping in view the guidelines given in IRC: SP-41-1994 and MoRT&H Designs for Intersections. In this stage of project, all these intersections are divided into two categories as major and minor, depending on their importance, and the necessary improvements have been proposed accordingly.

Table 4- 4: Details of Existing Junctions

| Sl No | Chainage (Km) | Junction Type | Junction Configuration | Direction |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | LHS | RHS |
| 1 | 12.200 | Major | 3 Arm | - | Devanahalli |
| 2 | 16.700 | Major | 4 Arm | Shidla | Hosakote |
| 3 | 26.600 | Major | 4 Arm | Chinthamani | Bangalore |
| 4 | 37.800 | Major | 3 Arm | Kaivara | Narasapura |

| Sl No | Chainage (Km) | Junction Type | Junction Configuration | Direction |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | LHS | RHS |
| 1 | 2.000 | Minor | 3 Arm | D. Hosur | - |
| 2 | 2.900 | Minor | 3 Arm |  | G.Hosahalli |
| 3 | 3.700 | Minor | 3 Arm |  | Gonuru |
| 4 | 3.900 | Minor | 3 Arm | Gollahalli |  |
| 5 | 6.300 | Minor | 3 Arm | Chinnondahalli |  |
| 6 | 6.700 | Minor | 3 Arm |  | Holerahalli |
| 7 | 13.200 | Minor | 4 Arm | Mithuna halli | J Venkatapura |
| 8 | 17.400 | Minor | 3 Arm | Hirabala | - |
| 9 | 18.600 | Minor | 3 Arm | Sandrahalli | - |
| 10 | 19.000 | Minor | 3 Arm | Kalyapura | - |
| 11 | 20.300 | Minor | 3 Arm | Mallanahalli | - |
| 12 | 21.800 | Minor | 3 Arm | Ralekunte | - |
| 13 | 22.800 | Minor | 3 Arm | Kalayanakana halli | - |
| 14 | 24.200 | Minor | 3 Arm | Ambiganahalli | - |
| 15 | 25.400 | Minor | 3 Arm | Kumbiganahalli | - |
| 16 | 28.400 | Minor | 3 Arm | - | Aarohalli |
| 17 | 28.800 | Minor | 3 Arm | Dalasagere | - |
| 18 | 29.800 | Minor | 3 Arm | Hanumantha pura | - |
| 19 | 31.000 | Minor | 3 Arm | - | Suladevanahalli |
| 20 | 33.000 | Minor | 3 Arm | - | Maligapanahalli |
| 21 | 41.600 | Minor | 4 Arm | Medihala | Andarahalli |
| 22 | 42.700 | Minor | 3 Arm | Bosanahalli | - |
| 23 | 43.500 | Minor | 3 Arm | - | Dhanamatnahalli |
| 24 | 44.400 | Minor | 4 Arm | Sagatur | Sulur |
| 25 | 49.200 | Minor | 4 Arm | Horahalli | Joohalli |

## 4.5   Land Acquisition

The proposed project road is widened symmetrically by retaining the existing centreline for most part. At the locations where the curves are improved and re-alignment is proposed, the proposed alignment will deviate from the existing centreline. Land should be acquired at those locations where the proposed alignment goes beyond the available RoW. Following table shows the area to be acquired at re-alignment locations.

Table 4- 5: Tentative Land Acquisition Details

| Sl No | Chainage (km) |  | Areas | Hectares |
| --- | --- | --- | --- | --- |
|  | From | To |  |  |
| 1 | 0.000 | 1.000 | 30000 | 3.0 |
| 2 | 1.000 | 2.000 | 30000 | 3.0 |
| 3 | 2.000 | 3.000 | 30000 | 3.0 |
| 4 | 3.000 | 4.000 | 30000 | 3.0 |
| 5 | 4.000 | 5.000 | 33000 | 3.3 |
| 6 | 5.000 | 6.000 | 33000 | 3.3 |
| 7 | 6.000 | 7.000 | 30000 | 3.0 |
| 8 | 7.000 | 8.000 | 30000 | 3.0 |
| 9 | 8.000 | 9.000 | 33000 | 3.3 |
| 10 | 9.000 | 10.000 | 33000 | 3.3 |
| 11 | 10.000 | 11.000 | 38000 | 3.8 |
| 12 | 11.000 | 12.000 | 35000 | 3.5 |
| 13 | 12.000 | 13.000 | 23000 | 2.3 |
| 14 | 13.000 | 14.000 | 25000 | 2.5 |
| 15 | 14.000 | 15.000 | 25000 | 2.5 |
| 16 | 15.000 | 15.200 | 4000 | 0.4 |
| 17 | 15.200 | 16.000 | 26400 | 2.6 |
| 18 | 16.000 | 17.000 | 27000 | 2.7 |
| 19 | 17.000 | 18.000 | 25000 | 2.5 |
| 20 | 18.000 | 19.000 | 23000 | 2.3 |
| 21 | 19.000 | 20.000 | 30000 | 3.0 |
| 22 | 20.000 | 21.000 | 27000 | 2.7 |
| 23 | 21.000 | 22.000 | 25000 | 2.5 |
| 24 | 22.000 | 23.000 | 25000 | 2.5 |
| 25 | 23.000 | 24.000 | 25000 | 2.5 |
| 26 | 24.000 | 25.000 | 15000 | 1.5 |
| 27 | 25.000 | 26.000 | 25000 | 2.5 |
| 28 | 26.000 | 27.000 | 25000 | 2.5 |
| 29 | 27.000 | 28.000 | 25000 | 2.5 |
| 30 | 28.000 | 29.000 | 25000 | 2.5 |
| 31 | 29.000 | 30.000 | 20000 | 2.0 |
| 32 | 30.000 | 31.000 | 25000 | 2.5 |
| 33 | 31.000 | 32.000 | 29000 | 2.9 |
| 34 | 32.000 | 33.000 | 30000 | 3.0 |
| 35 | 33.000 | 34.000 | 25000 | 2.5 |
| 36 | 34.000 | 35.000 | 0 | 0.0 |
| 37 | 35.000 | 36.000 | 0 | 0.0 |
| 38 | 36.000 | 37.000 | 0 | 0.0 |
| 39 | 37.000 | 38.000 | 0 | 0.0 |
| 40 | 38.000 | 39.000 | 30000 | 3.0 |
| 41 | 39.000 | 40.000 | 30000 | 3.0 |
| 42 | 40.000 | 41.000 | 30000 | 3.0 |
| 43 | 41.000 | 42.000 | 30000 | 3.0 |
| 44 | 42.000 | 43.000 | 23000 | 2.3 |
| 45 | 43.000 | 44.000 | 28000 | 2.8 |
| 46 | 44.000 | 45.000 | 29000 | 2.9 |
| 47 | 45.000 | 46.000 | 25000 | 2.5 |
| 48 | 46.000 | 47.000 | 30000 | 3.0 |
| 49 | 47.000 | 48.000 | 30000 | 3.0 |
| 50 | 48.000 | 49.000 | 30000 | 3.0 |
| 51 | 49.000 | 50.000 | 25000 | 2.5 |
| 52 | 50.000 | 51.000 | 25000 | 2.5 |
| 53 | 51.000 | 52.000 | 30000 | 3.0 |
| 54 | 52.000 | 53.000 | 30000 | 3.0 |
| 55 | 53.000 | 54.000 | 29000 | 2.9 |
| Total |  |  | 1393400 | 139.34 |

## 4.6 Existing CD Structures-

There are 23 CD Structures are located all along the Project Road. The details of CD structures are as follows.

Table 4- 6:  Details of Existing CD Structures

| Sl. No | District | Name of the Road | Type of CD structures |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Major Bridges | Minor Bridges | Pipe Culverts | Stone Culverts | RCC |
| 1 | Chikkaballapura Bangalore Rural Kolar | Vijayapura Nandi road via Devanahalli Kempapura (SH-96) | 10 | 9 | 5 | 13 | 28 |
| TOTAL - 65 Nos. |  |  |  |